# SIMATS ENGINEERING

## ASSESSMENT TOOL 1

COURSE NAME:	CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Annexure A

SHORT ANSWER TEST

Code of Conduct:

I Madhumitha R 192512274  (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Madhumitha R

Annexure A

Short Answer Test

- A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

- A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

- A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

- A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

- A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

- Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

- An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

- A packet experiences the following processing delays at different OSI layers: Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

- A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers. Calculate the total number of frames required, the total overhead

transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

- A hospital transfers a patient's medical record of 240 MB through an IoT-enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.

RUBRICS FOR CALCULATION

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understandin g of Concepts | 25% | Demonstrates a thorough, accurate understandin g of concepts | Good understanding with minor gaps | Basic understanding with some misconception s | Poor understanding; major misconception s |
| Explanation | 25% | Detailed, well-explaine d answers with relevant examples | Clear explanation with adequate details | Limited explanation; lacks depth | Poor or unclear explanation |
| Application | 20% | Effectively applies concepts with strong analytical ability | Applies concepts with minor errors | Limited application with weak analysis | No application or incorrect analysis |
| Organization | 15% | Well-structure d answers with logical flow and coherence | Organized with minor issues in flow | Basic structure, lacks clarity | Poor organization, incoherent answers |

| Accuracy | 10% | Completely accurate and covers all required points | Mostly accurate with minor omissions | Partially correct with missing points | Incorrect answers with major gaps |
| --- | --- | --- | --- | --- | --- |
| Clarity | 5% | Clear, neat, professional presentation | Understandabl e with minor issues | Limited clarity, readability issues | Poorly written, difficult to understand |

1. A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

Given:

1.File size = 150 MB = 150 × 10⁶ bytes = 150,000,000 bytes

2.Segment size = 1500 bytes

3.Data Link overhead = 40 bytes/frame

4.Bandwidth = 50 Mbps

Calculations:

1.Number of segments = 150,000,000 ÷ 1500 = 100,000 segments.

2.Number of frames = 100,000 frames (1 segment per frame)

3.Total overhead = 100,000 × 40 = 4,000,000 bytes (4 MB)

4.Total data transmitted = 150,000,000 + 4,000,000 = 154,000,000 bytes

5.Convert to bits = 154,000,000 × 8 = 1,232,000,000

6.Transmission time = 1,232,000,000 ÷ 50,000,000 = 24.64 seconds

Explanation:

Transport Layer divides the file into segments, numbers them, and ensures reliable delivery using acknowledgments and retransmissions.

Data Link Layer encapsulates segments into frames, detects errors using CRC, and retransmits damaged frames if necessary

2. A Transport Layer protocol uses a sliding window size of 6 frames. Each Frame carries 1024 bytes of user data. If the sender transmits 48 frames, the number of transmission windows required. If one frame in every Window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

Given:

1.Window size = 6 frames

2.Total frames = 48

3.Frame size = 1024 bytes

Calculations:

Transmission windows required = 48 ÷ 6 = 8 windows

One frame lost per window = 8 retransmissions

Explanation:

Flow control prevents the sender from overwhelming the receiver by limiting the number of unacknowledged frames, improving efficiency and reducing packet loss.

3. A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total Header overhead introduced. Explain how the Network Layer ensures successful Delivery of fragmented packets to the destination.

Given:

IP packet = 12,000 bytes

MTU = 1500 bytes

IP header = 20 bytes

Calculations:

Maximum data per fragment = 1500 − 20 = 1480 bytes

Number of fragments = ⌈12,000 ÷ 1480⌉ = 9 fragments

Total header overhead = 9 × 20 = 180 bytes

Explanation:

The Network Layer fragments packets to fit the MTU. Each fragment has its own IP header. At the destination, fragments are reassembled using identification, fragment offset, and the More Fragments (MF) flag.

4.A Transport Layer protocol uses a sliding window size of 6 frames. Each Frame carries 1024 bytes of user data. If the sender transmits 48 frames, Calculate the number of transmission windows required. If one frame in every Window is lost and must be retransmitted, determine the total number of Retransmissions. Explain how flow control improves communication efficiency.

Calculations:

Transmission windows required = 48 ÷ 6 = 8 windows

Retransmissions = 8 frames

Explanation:

Flow control regulates the transmission rate so that the sender transmits data only as fast as the receiver can process it, avoiding congestion and improving communication reliability.

5. A multimedia application transfers a 600 MB video file. The Session Layer Inserts a synchronization checkpoint after every 60 MB of transmitted data.During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the Amount of data that must be retransmitted after recovery. Explain the importance Of synchronization in the Session Layer.

Given:

File size = 600 MB

Checkpoint every 60 MB

Failure after 390 MB

Calculations:

Checkpoints before failure = 390 ÷ 60 = 6 checkpoints (at 60, 120, 180, 240, 300, 360 MB)

Last checkpoint = 360 MB

Data to retransmit = 390 − 360 = 30 MB

Explanation:

The Session Layer creates synchronization checkpoints so that after a failure, transmission resumes from the last checkpoint instead of restarting from the beginning, saving time and bandwidth.

6. Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the Compressed size. Calculate the compressed file size, the encrypted file size, and The total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

Given:

Original file = 900 MB

Compression = 40%

Encryption overhead = 5% of compressed size

Calculations:

Compressed file size = 900 × 60% = 540 MB

Encrypted file size = 540 × 1.05 = 567 MB

Percentage reduction = � = 37%

Explanation:

The Presentation Layer compresses data to reduce transmission time and encrypts it to provide confidentiality and security.

7.An FTP application transfers 3 GB of data over a network with an effective Throughput of 120 Mbps. The Application Layer divides the data into requests Of 3 MB each. Calculate the total number of requests generated and estimate the Total transmission time in seconds. Discuss how the Application Layer provides Reliable services to users.

Given:

Data = 3 GB = 3000 MB

Request size = 3 MB

Throughput = 120 Mbps

Calculations:

Number of requests = 3000 ÷ 3 = 1000 requests

Total data = 3000 × 8 = 24,000 Mb

Transmission time = 24,000 ÷ 120 = 200 seconds

Explanation:

The Application Layer provides user services such as FTP for reliable file transfer, authentication, error handling, and file management.

8.A packet experiences the following processing delays at different OSI layers:Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, And the transmission delay is 12 ms. Calculate the total end-to-end delay Experienced by the packet and explain how each layer contributes to the Communication process.

Given:

Application = 4 ms

Transport = 3 ms

Network = 5 ms

Data Link = 2 ms

Physical = 1 ms

Propagation delay = 18 ms

Transmission delay = 12 ms

Calculation: Total delay = 4 + 3 + 5 + 2 + 1 + 18 + 12 = 45 ms

Explanation:

Application Layer: User services.

Transport Layer: Reliable data delivery.

Network Layer: Routing.

Data Link Layer: Framing and error detection.

Physical Layer: Bit transmission.

Propagation delay: Time for signal to travel.

Transmission delay: Time to place data onto the medium.

9.A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different  OSI layer Calculate the total number of frames required, the total overhead

Given:

Useful data = 80 MB = 80,000,000 bytes

Frame size = 1600 bytes

Overhead/frame = 80 bytes

Calculations:

Useful data per frame = 1600 – 80 = 1520 bytes

Number of frames = 80,000,000 ÷ 1520 ≈ 52,632 frames

Total overhead = 52,632 × 80 = 4,210,560 bytes ≈ 4.21 MB

Total data transmitted = 80,000,000 + 4,210,560 = 84,210,560 bytes (≈84.21 MB)

Explanation:

Protocol overhead contains headers and trailers added by the OSI layers for addressing, error detection, and control, ensuring reliable communication.

10.A hospital transfers a patient's medical record of 240 MB through an Iot-enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame.Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.

Given:

Original file size = 240 MB

Compression = 30%

Segment size = 1400 bytes

Data Link overhead = 60 bytes/frame

Step 1: Compressed file size

Compressed size = 240 × (100 – 30)%

= 240 × 70% = 168 MB

Step 2: Number of segments

Convert compressed file to bytes:

168 MB = 168,000,000 bytes

Number of segments = 168,000,000 ÷ 1400 = 120,000 segments

Step 3: Total protocol overhead

Overhead per frame = 60 bytes

Total overhead = 120,000 × 60 = 7,200,000 bytes = 7.2 MB

Step 4: Final data transmitted

Final data = Compressed data + Overhead

= 168 MB + 7.2 MB

= 175.2 MB

Answer:

Compressed file size: 168 MB

Total segments transmitted: 120,000

Total protocol overhead: 7.2 MB

Final data transmitted: 175.2 MB